修正　2022.1.2６

Chapter 1 Introduction to Ground Improvement Techniques

Abstract: In this chapter, first the basic principles of Soil Mechanics are described, which are used in applying a particular ground improvement technique in the field. Some of the historical problems in Geotechnical Engineering are then described, which require a meticulous use of ground improvement techniques. Finally various ground improvement techniques which have been in use all over the world are classified according to their purposes and applications.

Keywords: Ground improvement purposes, Soil mechanics, Soil strength, Soil types, Problematic soils, Ground improvement types,

  - Prologue

Ground improvement is one of the basic requirements of many civil engineering construction projects. Improving the ground permits construction of structures on problematic soils and sites by changing the soil characteristics and strength. The main purposes of improving ground are as follows:

- Improve bearing capacity

- Reduce settlement of soft ground

- Prevent earthquake-induced liquefaction

- Control groundwater

- Stabilize bottom of excavation

- Prevent deformation of surrounding ground

- Prevent contamination of ground and cleaning up

Ground improvement can be achieved through three basic principles: Densification, Reinforcement and Solidification. Based on these principles eight basic ground improvement techniques are available. They are:

- Replacement, Consolidation and Compaction

- Consolidation and Drainage

- Compaction

- Solidification

- Drainage

- Water cut-off

- Load reduction

- Reinforcement

要検討

This book focuses on the principles of various ground improvement techniques developed in Japan, Asia and the rest of the world. The book will encompass the understanding of the problems facing geotechnical researchers and engineers working in the field of ground improvement, and will also cover the recent advances in the techniques available to them. The book consist of three parts. Part I focuses on the approaches for ground improvement geosystem dividing the each techniques in various groups and mechanics involved in each techniques. Part II describes many case histories from Japan and other Asian countries based on the authors‘ experiences of working in various ground improvement projects and their implementation. Part III focuses on the new applications and trend of the ground improvement techniques including recycling, geo-environmental consideration and preservation of the world cultural heritage.

In this book, various ground improvement techniques, their theoretical background, methods adopted in a particular situation and accomplishment of the projects are described in an easily understandable way. This book also helps students, researchers and engineers in understanding the basics of ground improvement, principles of the improvement methods considering the geological and geotechnical background, basis of selection of a particular ground improvement technique in a project, the execution of projects, and the cost-performance of such methods, etc.

This book will also benefit the beginners and the intermediate level practitioners who are dealing with geotechnical construction projects or who have interest in the development and application of ground improvement techniques. Engineers and researchers will find it helpful in developing, advancing and applying the techniques in the field.

1.2. Soil Mechanics Aided Ground Improvement Techniques

Structure and Composition of Soils.

Soil is an assemblage of particles, and, thus, there are separate volumes of solid and void. The voids are occupied with air and water. To define many key parameters in soil mechanics, soil assemblage with spaces of solid (grain) and void (air and water) is modeled into a three-phase diagram, as shown in Fig. 1.1 (Ishibashi and Hazarika, 2015). Volume side and weight side are drawn in three phases (solid, water, and air), and individual components are designated as Vs, Vw, Va and Ws, Ww, Wa. The total volume and the total weight are also designated as V and W, respectively. The volume of void Vv is the volume of water Vw plus the volume of air Va. Note that the weight of air Wa is assigned as zero since it is negligible relative to other weights of the element. The following definitions are made based on the three-phase diagram.

Porosity, n = Volume of voidsTotal volume=VvV=Va+VwV		(1.1)

Void ratio, e = Volume of voidsVolume of solids=VvVs		  (1.2)

The following relationships between void ratio and porosity can be obtained from the above definitions.

n=e1+e  or  e=n1-n			                              (1.3)

Water content, w = Weight of waterWeight of solids=WwWs(×100%)		         (1.4)

The water content for completely dry soil is 0 % and normally up to several tens % for fully saturated soils. However, the value may go up to more than 200 % for highly open-structured clay formed under marine environment and for organic soils as unusual cases.

Degree of saturation, S = Volume of waterVolume of solids=VwVs(×100%)		(1.5)

S value changes from 0 % for completely dry soil condition to 100 % for fully saturated soil. The soils with 0 < S < 100 % are called partially saturated soils. Note that in many occasions, “saturated” soils are interpreted as “fully saturated” without spelling out “fully”.

Specific gravity, Gs =Unit weight of solidUnit Weight of water=WsVsγw(×100%)	           (1.6)

where γw is the unit weight of water and is 9.81 kN/m3. Most soils have a rather narrow range of Gs values: 2.65 to 2.70. This implies that solid particle is about 2.65 to 2.70 times heavier than the weight of water for the same volume. If specific gravity test was not performed during the initial evaluation of geotechnical engineering problems, assuming Gs as a value between 2.65 or 2.70 would not produce a major error in the results.

There are several definitions of unit weights of soil.

Total unit weight, t = Total weight Total volume=WV=Ws+WwVs+Vw+Va 		(1.7)

Dry unit weight, d = Weight of solidTotal volume=WsV 		(1.8)

Note that γd is not necessarily physical dry unit weight of soils, but it rather is treated as a case by mathematically removing water while maintaining constant total volume V without shrinkage, which takes place during the physical drying process. Thus, for mathematically dried soil, the following equation is obtained.

t = (1+w)Gsγw(1+e)=(1+w)γd or d =Gsγw(1+e)= γt(1+w)		       (1.9)

This relationship is conveniently used to compare the effectiveness of compaction by the dry unit weight in compaction test analysis. The total unit weight does not measure the effectiveness of compaction directly, since the weight of water in void will increase the total unit weight, regardless of increase in void (less compaction) or decrease in void (more compaction).

The last important definition is the submerged unit weight γ' (or buoyant unit weight), which is the soil’s unit weight under water.

γ'= γt-γw= (Gs+Se)γw1+e-γw= Gs-1-e1-Sγw1+e for partially saturated (1.10)

In the last-term expression in Eq. 1.10, the degree of saturation S could be other than 1.0 (full saturated) even though soils are under water. It is possible shortly after soils were submerged in water. However, eventually, soils under the water table will be fully saturated (S = 1.0) and thus, it becomes

γ'= γt-γw= (Gs+e)γw1+e-γw= (Gs-1)γw1+e for fully saturated	(1.11)

Compaction of soils.

As seen in the first term of the γt expression in Eq. 1.9, increasing w increases γt for a same value of void ratio e, which is a measure of compaction effectiveness. Thus γt cannot be used to evaluate the effectiveness of compaction. On the other hand, the γd expression in the same Equation shows a direct relationship between “e” and “γd.” Accordingly, γd = γt / (1 + w) in Eq. 1.9 is used in the compaction analysis.

Fig. 1.2 shows the relationship between the dry unit weight and water content a soil which is a curve, called the compaction curve. In Fig. 1.2, γd increases with increase of w in the beginning, reaches the peak value (γd,max) at certain w, and drops thereafter. The peak of γd is defined as the maximum dry unit weight γd,max, and the corresponding water content is called the optimum water content, wopt. In the beginning, the addition of water works as a lubricant between particles to reduce the void and then to increase the dry unit weight. at different degrees of saturation of soils.

Fig. 1.3 shows a unique relationship between γd and w for a fixed S (degree of saturation) value and for a given Gs value. Fig. 1.3 also shows a group of curves from Equation 5.6 for various S values (40 %, 60 %, 80 %, and 100 %) with Gs= 2.7.

It can be seen that, when w increases, S increases. At γd max, S reaches more than 90 % and S approaches nearly 100 % (full saturation) when the water content passes wopt. The S = 100 % curve is called the zero air void (ZAV) curve, and compaction curves approach the ZAV curve at high water content as seen. Therefore, this curve is often used as a guideline to construct a proper compaction curve for a high water content zone.

Consolidation of soils.

It is important to know the difference between the process of compaction and the process of consolidation first. Fig. 1.4 shows the state of soils in (a) saturated, (b) unsaturated and (c) dry conditions. In compaction, the air is expelled, while in consolidation water is expelled. While the former is a must in any construction practices, the latter is undesirable in many cases. However, since consolidation of clayey soils is unavoidable under application of overburden load, in many construction practices forced consolidation is performed.

The relationships between consolidation stress σ (in log scale) and final void ratio e are plotted in Fig. 1.5. This curve is called the e-log σ curve and is a key relationship to determine final consolidation settlement. The loading curve (decreasing e with increase in σ) and unloading curve (increasing e with decease in σ) are seen in the figure. In the loading section, a linear relation is observed at higher stress level and a straight line is drawn as virgin compression curve. The virgin curve is the e and σ relations for naturally deposited soils on the bottom of lakes (or rivers), which are fully consolidated under their own gravity. The slope of the virgin curve is read as compression index Cc, and is given by

Cc = -(e-ei)logσ-logσi= -(e-ei)logσσi, thus, ∆e= ei-e= Cclogσσi 		(1.12)

where (ei, σi) and (e, σ) are arbitrary points on the virgin curve as seen. Eq. 1.12 is used to estimate consolidation settlement for soils, which follow only the virgin curve relation.

In primary consolidation theory and practice, there are two different computations: (1) amount of final consolidation settlement, and (2) time to reach a certain percentage of the consolidation. Therefore, it is practical and less confusing to separate consolidation problems into two categories: (1) how much, and (2) how soon (rate problem). By doing so, the selection of key consolidation equations, and the handling of the clay layer thickness (H or 2H) become easier. The following summarized these two different procedures.

“How Much” Problem

Depending on normally consolidated or overconsolidated, Eq. 1.13 or Eq. 1.14 are used, respectively.

Sf = H1+e0∆e= H1+e0Cclogσ0+'∆σσ0'  for normally consolidated soils	(1.13)

or

Sf = H1+e0∆e  for over consolidated soils	                     (1.14)

In Eq. 1.13 for normally consolidated soils, ∆e can be read directly from the e-log σ curve, which consists of only the virgin curve, or it is obtained from computed Cc, σ′0, and ∆σ values. Meanwhile, in Eq. 1.14 for overconsolidated soils, only the e-log σ curve is available to compute the final settlement. In both cases, H is the full thickness of clay layer regardless of top and bottom drainage conditions.

“How Soon” Problem

This problem always utilizes the relationship between the time factor Tv (Eq. 1.15) and the degree of consolidation U (defined as the percentage of settlement at an arbitrary time t to its final settlement).

Tv = CvtH2   								(1.15)

The settlement calculation model is shown in Fig. 1.6.

In this case, H is the longest drainage distance, and thus, if only one boundary layer is pervious and the other is impervious, H in Eq. 1.15 is the full clay layer thickness. On the other hand, if both the top and bottom layers are pervious, H in Eq. 1.15. is half the clay layer thickness.

Strength of Soils

Soil strength may be attributed to two distinctly different mechanisms of materials: one is its frictional resistance and the other is cohesive resistance along the shearing zone. As seen in Fig. 1.7, shearing of a soil assemblage in (a), which is subjected to normal stress and shear stress, is modeled with a block on a solid plate with a rough surface as seen in (b). In the model, shear stress τ is resisted by frictional mechanism and cohesive resistance between the interface of the block and the solid plate. Frictional resistance τfriction follows Coulomb’s friction law (τfriction = σ tanφ), where σ is the normal stress and φ is called the angle of internal friction of soil. The angle φ can be interpreted as the friction angle between facing soil elements along the shear surface. Cohesion resistance c is called cohesion of soil. In soils, normal stress independent cohesion comes from particle-to-particle close-range interactive forces, and it is a material property of fine particles (clays or cohesive soils).

Accordingly, the total shear stress at failure τf is expressed as

τf=c+ σtanφ 							(1.16)

Eq. 1.16 is a linear relationship between σ and τ and plotted as a straight line in any stress combination of σ and τ on any arbitrary plane (as shown in Fig. 1.8)

Eq. 1.16 is also called the Mohr–Coulomb failure criteria for determining the strength of soils, which is attributed from Coulomb’s contribution on frictional law and linear representation of its relation and Mohr’s contribution on defining failure with a unique combination of normal stress σ and shear stress τ.

Bearing Capacity of Soils

Terzaghi (1943) developed a bearing capacity solution for a continuous shallow foundation with a footing width B and an embedded depth Df under a level ground as seen in Fig. 1.9. Terzaghi obtained the following equation to determine the ultimate bearing capacity qu as

qu=cNc+γ1DfNq+ 12γ1DfNq 						(1.17)

where, Nc, Nq, and Nγ are bearing capacity factors and functions of the effective angle of internal friction φ' of the soil. γ1 is the unit weight of soil above the base of footing level and γ2 is the unit weight of soil below the base level. The cNc term is a contribution to the bearing capacity from the cohesion resistance along the failure surface, and it is zero for c = 0 materials (non-cohesive soils). The term γ1DfNq is a contribution from the surcharge load γ1Df at the footing base level, and it is zero for foundations placed on the ground surface. The ½γ2BNγ term is from the frictional resistance along the failure surface, and it is zero for φ' = 0 materials (cohesive soils).

1.3 Mechanism for increasing of soil strength

The strength of soils τ is represented by the cohesion c and the angle of internal friction φas described in Section 1.2. In addition, this strength will be dependent upon the confining pressure from the soil mass and its water content.

Fig. 1.10 (Ingles and Metcalf, 1972) shows a model diagram of strength increasing mechanism in soils. The vertical axis indicates the water content, the horizontal axis indicates the confining pressure. From this figure it is clear that the shear strength of soil increases as the water content is reduced. On the hand, the strength of soil increases as the confining pressure increases. The maximum gradient of soil strength increasing rate is realized when the confining pressure is increased and the water content is decreased at the same time.

Let’s consider the case of the consolidated pressure, instead of the confining pressure. When the saturated clay is being consolidated, its void ratio is reduced (due to decrease in water content) and as a result, the soil shear strength increases (Fig. 1.11). Fig. 1.11 (a) shows the related change between void ratios and consolidated pressures in the process of loading, unloading and re-loading of the consolidation test. Fig. 1.11 (b) shows the change of the shearing strength in relation to Fig. 1.11 (a). From Fig. 1.11 (a), when the consolidated pressure increases by Δ, point a moves to point b during the consolidation, and the void ratio will be reduced by an amount Δe. At the same time, the shearing strength cu increases from point a to point b in proportion to the consolidation

pressure p, as shown on Fig. 1.11 (b). In other words, when the consolidation process of soil goes on, its void ratio is reduced and its shearing strength is increased.

This phenomenon demonstrates the basic principle of the ground improvement techniques. In other words, if the consolidation pressure for soil mass is increased, its water content will be decreased and its density will be increased, and consequently the soil strength will be increased.

As described in Section 1.4 more in detail, the principles for increasing of soil strength can be categorized as “densifying of soil mass”, “solidifying of soil mass” and “reinforcing of soil mass”. Table 1 summarizes how the various improvement categories improve the ground.  Figs. 1.12 and 1.13 illustrate the mechanism for increasing soil strength. The strength increase is discussed from the viewpoint of cohesion, c and the angle of internal friction, φ. In both the Table 1.1 and Figs. 1.12 and 1.13, soil types are described as φmaterial (sandy soil), c-φmaterial (intermediate soil), and c material (clayey soil), because the functions of c and φ will be different depending on the soil types.

| Soil Type | Category of Ground Improvement |  |  |  |
| --- | --- | --- | --- | --- |
|  | Densification | Solidification | Reinforcement | Changing formation |
| ∅ soil (sand) | ∅: increase c: constant | ∅: constant c: increase | Apparent increase of ∅& c | Light weighting, Replacement, Counter weighting, etc. |
| c-∅ soil | ∅: increase c: increase | ∅: constant c: increase |  |  |
| c soil (clay) | c: increase | c: increase |  |  |

Fig. 1.12 schematically illustrates the change of c and φ for each soil type, due to densification of soils. Since the φmaterial, such as sandy soils, do not have c component, the increase of soil strength by soil densification is appeared due to an increase of φ only (Fig. 1.12 (a)). On the other hand, since c material, such as clay, which does not have φcomponent, the increase in shear strength of soil appears due to the increment of c, and it attains a constant value regardless of the confining pressure (Fig. 1.12 (c)). The intermediate soils are c-φ material, and their strength are results of the increments of c and φcomponents by the densifications (Fig. 1.12 (b)).

Similarly, Fig. 1.13 shows the change of c and φ, when soils have been solidified by adding chemical substances. Fig. 1.13 illustrates the change of c and φ for each soil type that are solidified by chemical materials (for example, inorganic solidifying materials such as Portland cement, or chemical polymer). Most chemical solidifying materials, which are used for ground improvement, generate adhesive force

Table 1.1 Soil type and mechanism of increasing soil strength

by chemical reactions. This adhesive force becomes the cohesion c, and the resulted increase in the strength of solidified soils is due to the increment of soil cohesion (Figs. 1.13 (a), (b), (c)). In many cases, basically, the increment of internal friction φ is not observed. However, when the solidified specimen is sheared, the decrement of internal friction may result, because some of the chemical materials may act as the lubricants.

In the case of “reinforced soil”, interaction between reinforcement and soil is by friction or mechanical interlock, which play the major role in improving the soils. Therefore, for the reinforced soil to work, the soil and reinforcement must strain, a precondition, in such type of ground modification technique. In a stable structure the strain in the soil and reinforcement are equal (i.e. there is strain compatibility). The strain in the reinforced soil is influenced by three factors, namely: The stiffness of the reinforcement, properties of the soil and the stress state of the soil.

As long as there is no slippage between soil and reinforcement, tensile stresses will be produced in the reinforcement and a corresponding compression in the soil element. It will be analogous to the existence of a pair of plates, which prevent the lateral expansion of the soil element as shown in Fig. 1.14 (a). The additional lateral pressure will move the Mohr circle to the right and away from the failure envelope as shown in Fig. 1.14 (b), and thus preventing the failure of the soil element (Narain, J., 1984).

On the other hand, Vidal (1978) describes reinforced earth as a cohesive material. The cohesion is assumed to be induced due to introduction of the reinforcement in an otherwise cohesionless soil. The anisotropic cohesion is produced in the direction of reinforcement, and the concept is based on the behavior of triaxial samples of reinforced earth.

1.4 Historical problems in Geotechnical Engineering

Leaning tower of Pisa

This famous building illustrates one of the historical problems in Geotechnical engineering. The 56-m-high bell tower at Pisa, Italy, leans about 3.97° or 3.9 m at top toward the south at present. The construction of the tower started in 1173 and was completed in 1372. It was reported that the tower started to sink after the construction progressed to the third floor in 1178 and more floors were built up to accommodate for the tilt. The lean is due to uneven settlement of the foundation soil caused by consolidation settlement of clay. In March 1990, the tower was closed to the public due to the possibility of collapse in the near future. Engineering remediation procedures were discussed to stop further leaning. The earlier attempt was made to put heavy load (800 metric ton of lead counterweight) on the north side of the tower foundation to compensate for the larger settlement on the south. A more drastic measure was taken later to extract soil mass (38 m3) under the north side of foundation soil by angled auger holes so that the north side experienced extra settlement. In December 2001, the tower was reopened to the public and has been declared stable for at least another 300 years (Fig. 1.15).

Kansai International Airport, Japan

Kansai International Airport near Osaka, Japan is a magnificent mega-construction project in recent decade with very challenging geotechnical engineering problems. The first-phase construction of this man-made island, 4.5 km long and 1.1 km wide, began in 1987, and the airport became operational in 1994. It was an amazingly fast-paced construction for this magnitude. The massive earth filling at an average water depth of 12 m on Osaka Bay required 208,000,000 m3 of reclaimed soil and rock (82 times the volume of great Giza’s Pyramid). The filling materials were brought from three excavated nearby mountains. Geotechnical engineers anticipated quite a large consolidation settlement due to this massive fill over a large area on soft bay foundation soil. Settlement, which immediately started, was carefully monitored, and results were compared with computed values. It sank 50 cm in 1994; settlement was reduced to 20 cm in 1999, and was 9 cm in 2006. Originally, engineers estimated 12 m total settlement in 50 years, but, in fact, it had already settled 11.5 m by 2001. Because of anticipated uneven ground settlement, the terminal building was equipped with jacks in each column so that uneven settlement could be adjusted not to have extra stress on individual columns. It is still sinking. Amazingly, the phase II runway, the second island on deeper water, has been constructed (Fig. 1.16) and opened to operation in 2007.

Liquefaction

Liquefaction is one of the major geotechnical engineering problems during earthquakes. It causes the settlement and tilting of buildings, catastrophic slope failures, and massive lateral movement of the ground.

During the 1964 Niigata Earthquake, Japan, with a Richter magnitude of 7.5, apartment buildings lost their foundation support and sank and tilted (Fig. 1.17). Foundation soil was transformed into viscous liquid due to earthquake vibration. Similar phenomenon was also observed in the Alaska earthquake that occurred in March 1964. Liquefied soil triggered massive landslides in Anchorage.

Soil liquefaction describes the behavior of soils that, when cyclically loaded, suddenly go from a solid state to a liquefied state, or having the consistency of a heavy liquid and cannot support foundation load any more. Liquefaction is more likely to occur in loose to medium dense saturated granular soils with poor drainage, such as fine sand or silty sand. During loading, usually cyclic undrained loading, for example, earthquake loading, loose sand tends to decrease in volume, which produces an increase in its pore water pressure and consequently a decrease in shear strength, that is, reduction in effective stress. Improving soils to mitigate liquefaction during earthquakes is one of the major challenge in any geotechnical engineering project in countries prone to earthquakes.

1.5 Types of ground improvement techniques

Originally, “Ground improvement” was termed as “Soil stabilization”. Ingles and Metcalf (1972) and Winterkorn et al. (1975) gave the overall and systematic description of “Soil stabilization”. In the State-of–the-Art report by Mitchell (1981), the ground improvement techniques were classified under 6 categories on the principles of methods. These are: in- situ deep compaction of cohesionless soils, pre-compression, injection and grouting; thermal treatment and reinforcement.

TC17 of International Society on Soil Mechanics and Geotechnical Engineering (ISSMGE) has proposed the classification system of ground improvement techniques, which is based on the broad trend of behaviors of the ground to be improved and whether admixture is used or not. Chu et al. (2009) described this classification system which consisted of the following 6 categories.

Ground improvement without admixtures in non-cohesive soils

Ground improvement without admixtures in cohesive soils

Ground improvement with admixtures

Ground improvement with grouting type admixtures

Earth reinforcement in fill

Earth reinforcement in cut

The classification system in this book is based on TC17’s classification, which is modified a little bit through the discussions and deliberations of the authors of this book. To describe it concretely, types of ground improvement techniques that fall under different groups are stated below.

Group A: Compaction, Vibro-compaction and Vibro-flotation, Dynamic compaction and consolidation

Group B: Replacement, Pre-loading using fill and vacuum, Vertical drain

Group C: Surface compaction using cement and lime, Sand compaction piles, Stone columns

Group D: Permeation grouting, Compaction grouting, Grout jetting, Mechanical soils mixing

Group E: Geosynthetics, Nailing and Anchoring

Group F: Freezing, Blasting, Electro-osmosis, Miscellaneous (Using natural and artificial materials)

Table 1.2 shows the various ground improvement techniques, the types of soil in which the techniques can be applied and the purpose that the special technique can serve. In the table, ◎means very effective, ◯means effective and △means conditionally effective. A blank in the matrix implies the particular technique is not suitable for the purpose stated.

Table 1.2 Ground improvement and ground control methods and their purposes

| Purposes Items (Ground Improvement Techniques) |  | Soil types to be adapted | Purposes of the Techniques |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Improvement of bearing capacity | Reduction of settlement | Reduction of earth pressure | Stabilizing natural or cut slope | Altering permeation | Prevention of sand liquefaction |
| 1. Ground Improvement without admixtures in non-cohesive soils or fill material |  |  |  |  |  |  |  |  |
| Group A | Compaction | Sandy soil | ◎ | ○ | △ |  |  | ○ |
|  | Vibro-compaction and Vibro-flotation | Sand | ◎ | ○ | ○ |  |  | ◎ |
|  | Dynamic compaction and consolidation | Sandy soil, gravel | ◎ | ○ |  |  |  | ◎ |
| 2. Ground Improvement without admixtures in cohesive soils |  |  |  |  |  |  |  |  |
| Group B | Replacement | Clay | ○ | ○ | ○ | △ | △ | ○ |
|  | Preloading using fill and vacuum | Cohesive soil | ○ | ◎ | ○ |  | ○ |  |
|  | Vertical drain (Sand and Gravel drain) | Sand・clay | ○ | ◎ |  |  |  | ◎ |
| 3. Ground Improvement with admixtures or inclusion |  |  |  |  |  |  |  |  |
| Group C | Surface compaction using cement and lime | Sand・clay | ◎ | ○ |  |  |  |  |
|  | Sand compaction piles | Sand・clay | ◎ | ○ | ○ | △ |  | ◎ |
|  | Stone columns | Sand・clay | ◎ | ○ | ○ | △ |  | ◎ |
| 4. Ground Improvement with grouting type admixture |  |  |  |  |  |  |  |  |
| Group D | Permeation grouting (Incl. compensation grouting) | Sand・clay | ◎ | ○ | △ | △ | ◎ | ○ |
|  | Compaction grouting | Sand・clay | ◎ | ◎ | ◎ | ○ | ◎ | ◎ |
|  | Grout Jetting | Sand・clay | ◎ | ◎ | ◎ | ◎ | ◎ | ◎ |
|  | Mechanical soil mixing | Sand・clay | ◎ | ○ | ○ | △ |  | ◎ |
| 5. Ground Improvement by earth reinforcement |  |  |  |  |  |  |  |  |
| Group E | Geo-synthetics | Sand, clay | ○ | ○ | ◎ | ◎ | △ | △ |
|  | Nailing and Anchoring | Sand・clay | ○ |  | ○ | ◎ |  |  |
| 6. Others methods of Ground Improvement |  |  |  |  |  |  |  |  |
|  | Freezing | Sand・clay | ○ | ○ | ◎ |  | ◎ |  |
|  | Blasting | Sand | ○ | ○ | △ |  |  | ◎ |
|  | Electro-osmosis | Sand・clay | △ | ○ | ○ | ○ | ○ |  |
|  | Miscellaneous (Using natural and artificial materials) | Sand・clay | Waste tires, glass, plastic, slag, wood, oyster shell, coconut fibers (coir) |  |  |  |  |  |

◎ Very Effective , ○ Effective, △ Conditionally effective

References

Chu, J., Varaksin, S., Klotz, U., and Menge’ P. (2009): Construction Processes, state of the art report. Proceedings of the 17th International Conference on Soil Mechanics and Geotechnical Engineering, pp. 3006-3135.

Hazarika, H. and Nakazawa, J. (2018): Classification of modern ground improvement techniques – A mechanism based approach - , Proceedings of the 13th Symposium on Ground Improvement, The Society of Material Science, Japan, pp.235-240.

Ishibashi, I. and Hazarika, H. (2015): Soil Mechanics Fundamentals and Applications, CRC Press, USA.

Ingles, O.J. and Metcalf, J.B. (1972): Soil Stabilization, Principles and Practice, Butterworths.

Mitchell, J. K. (1981): Soil Improvement, state of the art report. Proceeding of the 10th International Conference on Soil Mechanics and Foundation Engineering, pp.509-565

Narain, J. (1984): Reinforced Earth, Seventh Indian Geotechnical Society (IGS), Annual Lecture, Proceedings of the 26th Annual Conference of IGS, India, pp. 201-219.

TC 17 Report (2009): Technical committee No. 17 on ground improvement, International Society for Soil Mechanics and Foundation Engineering (ISSMGE), Retrieved from www.bbri.be/go/tc17

Vidal, H. (1978): The Development and Future of Reinforced Earth, Keynote Lecture, Proceeding of the Symposium on Earth Reinforcement, ASCE, Pittsburg, USA, pp. 1-61.

Winterkorn, H. F. and Fang, H. Y. (1975): Foundation Engineering Handbook, Van Norstrand Reinhold Co.

# Questionnaires – Practicing what you have learned.

1.1 How is soil a very unique material and complex in nature?

1.2 Summarize for the following questions on Ground improvement techniques.

1.2.1 What is the engineering purpose of adopting Ground improvement techniques?

1.2.2 What is the principle of Ground improvement techniques?

1.3 The engineering strength τ of soil is expressed by the cohesion c and the internal friction angle φ, but consider the effect by increments of c and φ according to the principle of ground improvement.