2022.1.20修正済

Chapter 2 Ground Improvement without Admixture in Non-cohesive Soils or Fill

Materials

Abstract: In this chapter, three techniques of Ground Improvement for non-cohesive soils /fill materials are described, in which soils are improved without adding any admixture. In principle, these methods can change the engineering properties of soils with relatively simple procedures. Especially, Compaction, Vibro-compaction and Heavy Tamping are described.

Keywords: Compaction, Filling, Reclamation, Vibro-compaction, Vibro-flotation, Heavy Tamping,

2.1 Introduction

This chapter describes Ground Improvement Techniques, in which soils are improved without adding any admixture in case of non-cohesive soils or fill materials. In principle, these method can change the engineering properties of soils with relatively simple procedures. Especially, Compaction, Vibro-compaction and Heavy Tamping are described. The principle of ground improvement is due to “Densification”, in which dynamic energy is imported to soils to change the engineering properties of the soil. However, it is to be noted that the improvement effects vary depending on the soil types. Since the principle of such ground improvement and the construction procedure are simple, it has been used as a method based on experiences of engineers from long ago. In this chapter, within this categories Compaction, Vibro-compaction and Heavy Tamping are described.

What is common to these methods is the addition artificial energy to soil masses in order to densify the soil mass. The energy is driven in this technique or dropping ―ーenergy of soil mass, static compressive energy and vibration energy. Soil masses to be improved are limited to non-cohesive soil and fill materials. When these energies are applied to cohesive soil and organic soil, they may lead to softening without improving the soil. The improvement method without admixture in cohesive soils (cohesionless) is described in Chapter 3.

Compaction is the process in which a stress applied to soil mass that causes densification as air and water are displaced from the pores between the soil particles. In modern ground improvement, this is achieved through heavy machinery compressing the soil. However, in ancient time, the same was achieved by allowing heavy animals such as also as work on the ground of heavy machinery compressing the soil.

The method of soil improvement whereby granular or sandy soils are compacted using special designed vibrators is known as Vibro Compaction or Vibroflotation. Naturally deposited soils as well as artificially reclaimed sands can be compacted from surface to great depths. The intensity of compaction can be varied in order to achieve the desired effect depending on the foundation or ground improvement purpose.

Heavy Tamping or Dynamic Compaction is a ground improvement technique whereby loose soils can be effectively and economically densified to improve its geotechnical characteristics and allow construction of structures directly on compacted soil, without need of deep foundations or soil replacement. The method involves dropping heavy steel weight repeatedly on the ground at regularly spaced intervals. The weight and height of tamping depends on the degree of compaction desired. The usual range of pounder weight is between 12 tf to 25 tf and the drop height can be up to 25m. This method is applicable for a wide variety of soil conditions including saturated/unsaturated loose sands, even with the presence of silty pockets, dune sands, inorganic fill, reclaimed soils with variable characteristics and sizes even with the presence of large sized boulders, landfill deposits and collapsible soils. This method has been extensively used to compact loose soils to depths of up to 10m, in order to increase the bearing capacity, decrease post construction settlement and mitigate liquefaction risk in case of seismic events.

2.2 Compaction

2.2.1 Description

When road banking, levee embankments, earth fill dams and land reclamations for residential area or factory sites are constructed newly, it is required that qualities of their earth structures are stronger, denser and more stable.

To satisfy those requests, generally suitable soil materials are compacted by artificial procedures generally. The concepts, by which the above-mentioned effects are achieved through compaction of soil materials, are described in Section 1.2 Soil Mechanics Aided Ground Improvement Techniques ("Compaction of soils" Fig. 1.2 and 1.3). If soil materials with suitable water contents are compacted, the void ratio of soil mass is decreased and the dry density is increased. As a result, the engineering properties of soil mass will be improved.

That is;

- The shearing resistances (c, φ) and California Bearing Ratio, etc. will be increased.

- It prevents engineering troubles such as the compression settlement and the submerged settlement of soil structures.

- When the seepage control effect for soil structures is intended such as the core of earth fill dams, the permeability can be lowered.

2.2.2 Principle and Mechanics

Fig.2.1 shows the relation between compaction curve and unconfined compression strength, coefficient of permeability and shear strength (c, φ) which corresponds to it. The maximum value of unconfined compressive strength is realized on the dryer side of the maximum dry density. The minimum value of coefficient of permeability is realized on the wetter side of the maximum dry density. Maximum angle of shearing resistance, φ is realized at maximum dry density and it decreases along the wetter side of water content. Cohesion, c also shows a maximum value on the dry side of optimum water content and decreases on wet side

Factors, which dominates the quality of compaction, are soil materials properties, water content and the compaction energy (refer to Fig.2.2). Relationship between the compaction energy and the compaction curve is shown in Fig.2.3. After the compaction curve for a given soil is obtained from laboratory test, the specification of compaction in the field is made. Relative compaction (R.C) is define as,

R.C＝γd,field /γd,max (×100)                      (2.1)

where γd,field is the specified dry unit weight, that shall be achieved in the field, and γd,max is the maximum dry unit weight obtained from the laboratory compaction test.

When constructing soil structure based on above-mentioned soil compaction properties, the concept of workable water content is explained in Fig.2.4. In this figure, a soil sample is tested by the standard Proctor test, and the compaction curve obtained is shown. The specification says the in-situ soil shall be compacted with 95% or more off. The range of field water content to achieve the above mentioned specification will be from w1 to w2, which can called as the water contents range for constructible earth-works. When the water content of soil material is lower or higher of the desirable water range in the field, water content shall be adjusted.

2.2.3 Procedure and Design

Since designing of soil structures is related closely to the construction method, procedures and designing of these using compaction are described in this subsection. Fig.2.5 shows the work flow inducing design consideration, the investigation of soil materials and the field parameters. The work on the jobsite consist of carrying soil materials, spreading and laying, and compacting with appropriate moisture content.

Construction machineries for compacting differ depending on soil types and earthwork mass. For small jobs such as filling excavated small trenches, hand-operated vibratory tamper (Fig.2.6 a) may be used. For larger job sites, several different types of heavy-duty compaction rollers are available. The commonly used compaction equipment are

Pneumatic rubber tire rollers (Fig.2.6 b): The pneumatic rubber tire roller can be used for both sandy soils and as clayey soils. Soils are compacted with both tire pressure and kneading action.

Sheep’s-foot rollers (Tamping rollers) (Fig.2.6 c): This unique wheel surface can effectively compact the clayey soils and deeper part of soils in earlier passes.

Smooth-wheel (drum) rollers (Fig.2.6 d): This is mostly used for both sandy and clayey soils for the finishing and soothing process. The compaction pressure is not as high as in the former two rollers, and, therefore, it is not used to compact thicker layers.

Vibratory wheels: All the foregoing rollers are usually equipped with vibratory wheels, which are especially effective in compacting granular soils.

There are several key parameters that influence field compaction in addition to the level of compaction energy and controlling the water content as close as possible to its optimum water content. They are as follows:

Number of passes: In general construction practice, several or more passes ofrollers are required to obtain a specified unit weight. The more number of passes is applied, the higher unit weight is obtained. Fig.2.7 plots γd versus depth with various number of passes from 2 to 45 of a single 2.44m lift fill by 55.6kN smooth roller (D’Appolonioa et al. 1969). After five passes, a large increase is needed in the number of passes to achieve significant increase in compaction. In general, it is considered that more than 10 to 15 rolls may not be effective and not economical way to compact fill.

Amount of fill: The amount of lift is also significant. Fig.2.7 also shows that only the upper section at 0.3m – 0.5m deep is effectively compacted. The lift should be small enough to get the maximum compaction effect over the entire depth, but not be too small, since the very top portion of the layer also cannot be well compacted because of particle segregation upon vibration application. In general applications, a loose lift is limited to about 0.5m.

2.2.4　Applications

Applicable examples of banking structure are shown in Table 2.1.

Table 2.1 Typical Application List

| Name of Filling Structure | Objectives | Function |
| --- | --- | --- |
| Road Embankment Railway Embankment | Traffic-ability | High Bearing Capacity Low Compressibility Sufficient Stability |
| Land Fill for Residences etc. | Bearing and Supporting |  |
| River Embankment Coastal Dike Storm Water Reservoir Earth-fill Dam | Seepage control Flood Control Water Storage | Low Permeability Low Compressibility Enough Slop Stability |

2.3 Vibro-compaction and Vibro-flotation

2.3.1. Loose sandy ground and vibration

The unsaturated sandy grounds by natural sedimentation or piling sandy soils are often in the loose state, because there are the apparent cohesion from meniscus actions of surface water and frictional arch actions between sands particles. If vibrating action is applied for these loose sandy masses, the above-mentioned inter-particle forces will be disappeared, and sand particles will settle at the position that the potential energy and the contact angles between particles are lower than former state. In the other words, the loose sandy soil mass has a tendency to be compressed easily when vibrating force is applied.

When the sandy soil mass is saturated by water (pore water), each sand particle acted in the state of buoyant force and the soil mass state is retained in the loose state. If the vibrating and loading is applied to the soil mass, it is compressed and the pore water is increased in pressure. The water pressures may be built to an extent where they exceed the contact stresses between the particles of soil, which keep them in contact with each other. This loss of soil structure causes to lose all of its strength (the ability to transfer shear stress) and the soil mass may be observed to flow like liquid (hence ‘liquefaction’). As pore water pressure rises a progressive loss of strength of the soil, the effective stress is reduced. On reaching a state of zero effective stress such soils immediate dilate. After then, if these soil mass is placed in that position, the soil mass will be more dense state than original and regain its strength. Fig.2.8 shows the above-mentioned states by diagrams. Therefore, the vibrating action with water-jetting is more effective for the improvement of sandy soils

Vibro-compaction, Vibro-flotation, Sand compaction pile and Stone column are listed typical ground improvement techniques which apply these principles. The former 2 methods intend for improving of shallow soil layer, and are based on concepts of uniform improvement of sandy ground. The latter 2 methods intend for improving  deeper soil layers, and based on concept of composite ground with strong piles. In this chapter, the former 2 methods are described and the latter 2 methods are described in Chapter 4 (Section 4.3 and 4.4).

2.3.2. Vibro-compaction

Details on Vibro-compaction are described in the preceding Section 2.2.

Vibro-compaction is a ground improvement technique that densifies clean, cohesion-less granular soil by means of vibrating action and vibrating loading. As for the improvement of the loose sandy soil, the way of the raising density of soil mass and the increasing strength of it are generally taken by adding vibration or percussion. The vibratory energy reduces the inter-granular forces between the soil particles, and allows them to move into a denser configuration. As a result of this procedure, the soil mass will be achieved relative densities of 70 to 85 percent. It will be increased on the internal friction angle and the stiffness. The typical construction machineries for this method are listed at Table 2.2, whose machineries are used at ground surface or layer’s surface of its spreading.

Table 2.2 Construction machineries for vibrating compaction

| Type | Name |
| --- | --- |
| Roller Hand-operated type Vibrating plate type | Flat roller, Tamping roller, Hand-guide roller Tamper, Rammer Vibro-plate, Plate compacter |

2.3.3. Vibro-flotation

Vibro-flotation method is an improving technique for loose sandy grounds by mechanical vibration and water jetting, and high densities in sandy ground will be obtained by this method. Vibro-flotation is carried out a special probe down into the sand deposit, that the probe is called Vibro-flot. The vibro-flot is a cylindrical tube with water jetting parts at top and is equipped with a rotating eccentric weight which develops a horizontal vibratory motion. By combination of mechanical vibration and water saturation, a quick sand state is created in the sand adjacent to the probe. The sand at a short distance from the probe is densified by compacting action. After then, the crater is generated around of the probe, and the additional sand or gravel is added to the crater from the surface as the probe is slowly withdrawn.

Vibro-flotation was developed in Germany and the Bureau of Reclamation studied the possibilities for sand and silt compaction at the site of the Enders Dam (D’Appolinia 1954). In Niigata earthquake, it was proved that the method was effective on the prevention of sand liquefaction in case of the earthquake (1965, Watanabe).

Fig.2.9 shows the range of soil grain size distribution suitable for compaction by Vibro-flotatoin.

Fig.2.10 shows the procedure of vibro-flotation method. First, jetting water is injected from the tip of vibro-flot to loosen the sandy ground. While the objective sandy ground is saturated with jetting water and is on the liquefied state, the effect of vibrating energy will travel the ground around the vibro-flot and compact it. As a result, the ground around the vibro-flot is compacted, the gap is completed between vibro-flot and the ground. In the gap, course grained materials (sand, crushed stone, gravel etc.) will be filled up and compacted. Finally, the column of the course grained materials with diameter 50-60㎝ is developed and there is an effect on the increasing density of the sandy soil ground and the decreasing compressibility of it. Probe spacing of compaction holes should be on a grid pattern of about the distance of 1~2m to produce relative densities than 70 percent over the entire area, and the improvement depth is equal to or less than 8.0m. End results are: increased shear strength, increased stiffness, and reduced liquefaction potential. Table 2.3 shows the ordinary and typical specification of Vibro-flot (Unitized type).

Table 2.3 Specification of Vibro-flot

| Items | Ordinary type (20H.P.） |
| --- | --- |
| Diameter (㎜) | 410mm |
| Length（ｍ） | 12.5m |
| Mass (㎏) | 5,400kg |
| Electric motor (kw) | 15kw |
| Frequency (rpm) | 1,450～1,740rpm |
| Vibrating force (tf) | 2.7～4.0tf |
| Note | Unitized type With water-jetting device Attachable for Vibro-hammer at the top |

The quantity of course grained materials which must be replenished for the ground improvement is calculated by the following formula.

(2.3.1)

whereas:

v: quantity of additional materials per unit volume (m3/m3)

e0: void ratio of original ground

e1: void ratio of additional materials

e’: necessity void ratio of ground after improvement

It estimates the ground strength after improvement from the relation between the internal friction corner and the relative densities. Table 2.4 shows the relationship between the relative density and the internal friction of sand.

Table 2.4 Relationship between relative density and internal friction

| Relative density | Internal Friction (φin degree) |  | N-value by SPT |
| --- | --- | --- | --- |
| (Dr %) | By Terzaghi | By Meyerhof |  |
| 0 〜 20 20 〜 40 40 〜 60 60 〜 80 80 〜 100 | Under 28.5 28.5 〜 30 30 〜 36 36 〜 41 Over 41 | Under 30 30 〜 35 35 〜 40 40 〜 45 Over 45 | 0 〜 4 4 〜 10 10 〜 30 30 〜 50 Over 50 |

There are many on oil tank basics, on architecture basics in the practical example.

Next, the pit, the road, the railroad and the runway at the factory and the warehouse and so on become the surface ground reinforcement.

Fig.2.11 shows an application of ground improvements by Vibro-flotation method.

It was planned to dredge the bottom soils at the existing wharf in order to facilitate docking of the large-sized ships to dock. Under its’s consideration and it’s designing, if the bottom of sea bed at front of the wharf is made to 1 m deeper than the present state, it concluded that the wharf will become unstable in case of the earthquake.

At a result of the reviewing, it was decided to apply ground improvement by Vibro-flotation method for the purpose of reducing the active earth pressure against the wharf and also preventing the liquefaction of sandy ground. The necessary condition for the ground improvement is to increase the average N value 7 of untreated ground to about 16. As shown in Fig.2.11, the depth of the ground improvement was set to 6.5 m on the sea side, and 2.9 m for the anchoring wall side. The specifications of ground improvement are as follows:

Arrangement of driving: 1.75 m triangle pattern,

Used machine: 30 HP Vibro-Flot,

Aggregate grade: Diameter range 5 – 40 mm

Quantity of aggregate: Sea side 2.0 m3 per one point

Anchoring side 1.3 m3 per one point.

Due to the improvement, the average N value between each driving point increased up to 21, and the objective was achieved.

2.4 Heavy tamping (Dynamic compaction)

2.4.1 Description

Heavy tamping is soil compaction by means of multiple drops of a heavy weight from a large height, which involves dropping heavy weights repeatedly on the surface of non-cohesive or cohesive soil deposits at regularly spaced interval, thus inducing shock waves, liquefaction, surface cracking and densification for these deposits. Sometimes this method is called as “Dynamic consolidation” or “Dynamic compaction”. The development of this method into a modern and efficient tool for ground compaction is mainly due to Menard et al. (1975). The technique has been used to densify a wide variety of soils, from organic and clayey silt to crashed rock, fill with a high content of large rock fragments. The improvable depth of the ground with this method will be estimated from 10m to 20m, because the impact energy per one blow is larger than other usual compacting methods. The ground to be improved is crashed stone, gravel, sand, cohesive soil and waste etc.

The weight is typically between 80 and 360kN, and the height changes from 10 to 30m. Usually, the weight mass is made of steels or reinforced concrete with its base area from 3 to 7m2. As for the operation of this method, the above-mentioned weight is lifted with the large-sized crane and is dropped repeatedly and freely from designated height. It is usual, to control the efficiency of compaction with pressure meter tests, obtaining the deformation moduli of substratum before and after compaction and also with long-term water pressure measurement.

2.4.2 Principle and designing

The principle of Heavy tamping is the same as conventional compaction method (Section 2.3). In addition, in saturated ground, the increase and the dissipation of pore water pressure contribute to the densification of the ground. When the impacting power is acted to the saturated ground, the following phenomena are occurred in the ground:

- At dropping point of the weight, the crater at the ground surface will be happened at once.

- At the same time, the excess pore water pressure in the ground occurs and remains after dropping of weight.

- By repeatedly dropping operation by the weight, the excess pore water pressure is increased.

- Excess pore water pressure and impacting power dislocate soil matrix.

- After settling, soil matrix rearranges structure towards denser state.

Fig.2.12 (Woods, 1968) shows these processes and phenomenon.

Fig.2.13 shows the illustrated system and procedure of Heavy Tamping.

Fig.2.14 (Yamada, 1993) demonstrates the above-mentioned phenomenon by the field monitoring, which shows the ground settlements, change of excess pore water pressure and soil strength by consecutive tamping. After tamping, the ground settlements were observed at once, which meant the decrease in void ratio. At the same time, the excess pore water pressure was occurred and remained after tamping. But it in the sandy ground disappeared at short time, and it in the clayey ground remains after tamping. When the excess pore water pressure dissipates, the soil strength increases to values above those existing before tamping.

Fig.2.15 shows designing conditions and specifications for Heavy Tamping, which should be decided prior caring out of it.

1) Mass of hammer, dropping height of hammer and improvement depth by tamping

As a rule of thumb, the depth of consolidation d, in meters, reached by this method can be estimated at roughly (Menard et al., 1975):

(2.4.1)

whereas  m; mass, in tons, of the falling mass

h; height, in meters, from which the mass is dropped.

As the case histories are increased, the following empirical formula is proposed, which has incorporated the coefficient with considering of objective ground (Yamada, 1991):

(2.4.2)   (Refer to Fig.2.13)

whereas   α: empirical coefficient,

(0.4-0.6 for sandy soil,

0.5-0.7 for crashed stone and gravel,

0.3-0.5 for cohesive soil and waste)

2) Location of dropping point

In the first step, the dropping point interval at ground surface should be designed the same as improved depth to be estimated. In the following step, it should be planned by the halfway point in the location of the pre-step. With the repeat of this procedure, it should be planned for a surface to be covered by dropping.

3) Dropping number

Dropping number of hammer is estimated by the following formula:

(2.4.3)

whereas   N: dropping number

E: total impacting energy (tf･m/m2) (Table 2.5)

l: space of dropping point (m)

W: weight of hammer (tf)

H: dropping height of hammer (m)

Table 2.5 Impact energy by soil condition (In the case of improved depth D=20m)

| Soil condition | Target |  | Ev | E |
| --- | --- | --- | --- | --- |
| Sandy or gravelly soil | Increment of N value by SPT | ΔN=5 ΔN=10 ΔN=15 | 18-23 35-45 60-70 | 360-460 700-900 1,200-1,400 |
| Waste | 30% volume reduction |  | 80 | 1,600 |
| Sandy soil with clay |  |  | 60-80 | 1,200-1,600 |

[Note]  Ev : Impact energy to be necessary at a unit volume(tf･m/m3)

E : Total impact energy to be necessary at a unit area (tf･m/m2)

Example problem:

?????

2.4.3 Procedure

As shown in Fig.2.13, this method consists of the simple devices (lifting machinery and weight) and the simple procedures in principle. Fig.2.16 shows the procedure flow of Heavy Tamping. Attending items in the executing process is as follows;

- Phasing and series

The tamping energy is applied to the ground in several blows for each tamping position and in several passes, following a preset grid (usually a square grid), split into the primary grid (P) and the secondary grid (S) (Fig.2.17. (In some cases, the third grid and the more grid should be located).

- Monitoring and testing

In Fig.2.16, “Decision”, which is represented as a diamond, should be determined by the improvement effect.

2.4.4 Application

As mentioned in TC17 (2007.09.05), Heavy Tamping has two major applications as follows:

* Densification of saturated or unsaturated natural sands as well as hydraulic fills for bearing capacity increase, settlement reduction or liquefaction mitigation.

* Densification of unsaturated miscellaneous soils in particular fills containing large particles (i.e. rocky fractions).

* For saturated soils, the process becomes marginal if the fine contents do exceed 35%, pending on the clay fraction.

Also, this method is applied to the stabilized reduction for waste dumps, sanitary landfills and mine waste.

Recent and large applications for sea side airport construction are described below as two examples:

Changi airport in Singapore

About 40 million cubic meters of sand was dredged for a 700 ha major hydraulic fill reclamation completed in 1976 for the construction of the Singapore Changi International Airport which was underlain partly by thick deposits of soft marine clay.

For the construction of the second runway of Changi airport, 7 m of hydraulic fill was placed over varying layers of both old and recent marine clays. Settlement of these clays could amount to several meters over a ten year period. The combination of dynamic compaction and vertical drains up to a depth of 43 m (a first at the time) was applied (Menard-Group pamphlet).

Kansai international airport

This airport was made through reclamation and with gravelly soils which contains rocky fractions of maximum grain size 300 mm, and heavy tamping was carried out for improvement of the reclaimed land. After the ground improvement, N value recorded more than 30 from the ground level up 8 m depth and 15 below 8 m depth (Nihon Kokudo Kaihatu Co. Ltd. Pamphlet).

References

D’Aoppolonia, E. (1954); Loose Sands – Their Compaction by Vibroflotation, Symposium on Dynamic Testing of Soils, ASTM Int, pp.138-162.

D’Appolonia,D.J., Whitman, R.V., and D’Appolonia, E.D. (1969): Sand compaction with vibratory rollers, Journal of Soil Mechanics and Foundation Division, Vol.95, No.SM1, pp263 – 284.

Hansbo, S. (1978): Dynamic consolidation of soil by a falling weight, Ground Engineering, July, 1978, pp.27～30.

Ishibashi. I., and Hazarika, H. (2010): Soil Mechanics Fundamentals, CRC Press, p.###

Lambe, T.W. (1958): The structure of compacted clay, Proc. ASCE, Vol.84, SM2, pp.

Menard, L. and Broise Y. (1975): Theoretical and Practical aspects of dynamic consolidation, Geotechnique, Vol.25, No.1, March Vol.25, No.1, March. p3～18.

Meanard-Group pamphret

Nakazawa, J. (1991); Vibro-flotation Method, Manual of Ground Improvement, Chapter 18, pp.343～352, Edit. Committee on Soil Stabilization (Soc. of Material Sci., Japan), Nikkan Kogyo Shinbunsha,. Japan (In Japanese).

Proctor, P.R. (1933): Four articles on the design and construction of rolled earth dams, Engineering News Record, Vol.111, pp.245 – 376.

Watanabe, T. (1965); Effects of Vibro-flotation method on Niigata Earthquake, Soils and Foundations, Journal of JSMFE, Vol.13, No.2, pp.27～33, (In Japanese)

Yamada, M. (1991): Dynamic consolidation, Chapter 19, pp.353～366, Edit. Committee on Soil Stabilization (Soc. of Material Sci., Japan), Nikkan Kogyo Shinbunsha,. Japan (In Japanese).

Questionnaires – Practicing what you have learned.

2.1 When a ground is improved by Compaction method, what are the changing factors of engineering properties of the compacted ground?

# 2.2 Why we should draw the zero air voids curve along with the compaction curve?

# 2.3. What are the two most important factors that affect the quality control during field compaction?

# 2.4. If a course grained soil contains more than 15% silt, will vibroflotation be effective to densify this soil?

# 2.5 During the dynamic compaction method, if the mass of the weight is 15 kg and the drop height is 20 m, calculate the compaction depth achieved.

# 2.6 Would you recommend to perform dynamic compaction method in a high density residential area?