Chapter 3 Ground Improvement without Admixture in Cohesive Soils

Abstract: In this chapter, the techniques for Ground Improvement for cohesive soils are described, in which the improvement can be achieved without using any admixtures in the soils. The improvement principle involves artificial utilization of the built-in geotechnical properties of soils. Replacement, Preloading using fill and vacuum, Vertical drain come under this category of ground improvement.

Keywords: Cohesive soil, Replacement, Preloading using fill and vacuum, Vertical drain, Consolidation.

3.1 Introduction

This chapter describes the techniques for ground improvement for cohesive soils, in which the improvement can be achieved without using any admixtures in the soils. The improvement principle involves artificially utilization of the built-in geotechnical properties of soils. Replacement, Preloading using fill and vacuum, Vertical drain come under this category of ground improvement.

Replacement is one of the oldest and simplest methods which improves the bearing capacity of foundation soils by simply replacing the original soils. The foundation condition can be improved by replacing poor soil (eg. organic soils and medium or soft clay) with more competent materials such as sand, gravel or crushed stone. The use of replacement soil under shallow foundation can reduce the consolidation settlement and increase the soil bearing capacity.

As explained in Chapter 1 (Fig. 1.10), when a confining pressure is applied to saturated soils, the pore water pressure rises and the soil water attempts to drain to the side where the water pressure is low. As a result of soil water drainage, the water content of soils deceases, accordingly its volume decreases, and the engineering properties of soils will be improved. Preloading (Section 3.3) and Vertical drain (Section 3.4) are two methods which are intended to effectively utilize these properties.

Preloading is the ground improvement method to promote consolidation settlement of the soft soils and to increase its strength by preloading prior to the construction of the structure The magnitude of preloading pressure usually rages from 1.2 to 1.3 times of the structure loads. Earth fills, water lowering, vacuum under impervious membrane are some of the techniques used for applying the preload. Once the settlement under the preload is completed, the load is removed and the construction of the structure is started. Through the use of Vertical drains, the consolidation process can be accelerated, thereby, reducing the time of Preloading. Vertical Drains, also known as Sand drains, Wick drains or Band drains, are used to accelerate the consolidation of embankments built in fine grained soils.

3.2 Replacement method

3.2.1 Description

The purpose of Replacement method is to maintain the stability and prevent the settlement of the superstructures by removing soft soil and replacing those with good quality soil materials. Generally, replaced materials are sand, gravel and crushed stones. In situations where the disposal site for soils, which have been excavated and removed, is unsecured, the excavated soils are mixed with Portland cement etc., and the stabilized soils are reused as replacing materials.

This method has been applied for the ground improvement since the olden times. However, if a large-scale construction is involved, sometimes the cost of this method turns out to be higher than other ground improvement methods. Replacement of the existing unsatisfactory soil is always a possibility within the obvious limitations. It may prove to be the most satisfactory solution for poor surficial soils of limited depth.

3.2.2 Principle

The principle of Replacement method is to replace the soft clay layer with large compressibility and little shear resistance to materials with low compressibility and high shear resistance to reinforce against subsidence of the foundation, lack of bearing capacity, and failure by shearing. The soft clay, which has high compressibility and low shearing resistance, will be removed, and the suitable soil, which has low compressibility and high shearing resistance, will be replaced. As a result, the settlement of the foundation will be prevented and the bearing capacity of soil layer will be increased. Replacement methods involve three procedures: Replacement, Displacement and Explosive replacement as shown in Table 3.1 (Chu et al 2009). The advantages and disadvantages of each method are also indicated in Table 1.

Table 3.1 Ground improvement by Replacement

(Chu, et al 2009)

|  | Description/mechanism | Typical applications | Advantages | Limitations |
| --- | --- | --- | --- | --- |
| Replacement | Ground is improved by removing poor soil and replacing it with suitable materials. Lightweight materials can be used backfill to reduce the load or earth pressure. | The method can be used when the area to be improved is limited and when only top few meters of soil needs to be improved. | 1) It can be applied to all types of soil that can be excavated easily. 2) Immediate improvement is achieved. 3) The bearing capacity and settlement of the soil can be controlled. | The method is expensive and limited to shallow depth of 3 to 4 m only. |
| Displacement | Soft soil is improved by using good soil to displace soft soil without removing the soft soil completely. | This method is suitable to soft, swampy area where excavation is difficult and when the depth of soil to be improved is limited. | 1) It is less expensive than the replacement method. 2 )It can be used when the soil to be replaced is very soft or highly organic such as muck and peat. | Soft soil cannot be completely replaced. Some soft soil pocket exist. Therefore, quality control can be difficult. |
| Explosive Displacement | Explosive is used to remove soft soil and causes a pile of rock to fall into the cavity created by blasting. | It can be used when the soft soil to be removed is less than 8 m and gravels, rock or crushed stones are available. | 1) It is less expensive than the soil replacement method. 2) The soil replacement ratio is higher than that achieved by soil displacement method. | 1) A relative complete replacement can only be achieved for the top soil depending on the position of the explosives. 2) It is not suitable to urban construction. 3) Quality control can be difficult. |

3.2.3 Procedures

(a) Replacement

Fig. 3.1 shows an example of road embankment in a soft soil improved by Replacement method (Nishida and Nakazawa, 1991). The excavation work for soils was carried out by using Drag-line, Back-hoe and Motor Scraper etc. Backfilling work and transporting work of replacing materials were carried out by dump trucks or bulldozers. Compacting work was carried out using the usual compacting rollers.

Similarly, Fig. 3.2 (Nishida and Nakazawa, 1991) shows the case of an underwater construction of breakwater foundation by Replacement method. In such constructions, usually dredging work for the concerned soils is carried out by using grab dredger or  pump dredger (including cutter pump), and backfilling work is carried out by hopper barge. Replacement materials to be used should have the sufficient engineering properties without compaction, because underwater compaction is difficult to perform. Fig. 3.3 shows an example in which lightweight material was used as replacement materials.

In certain situations, a practical possibility is to remove the underlying soft soil layer by instigating shear failure in the layer. To produce that, a sufficient surcharge is applied to squeeze out the soil, and thus producing an artificial mud flow or an artificial slip failure. Fig. 3.4 shows an example of Replacement by surcharging and squeezing, in which a forcible lateral extrusion and replacement the soft clay layer could be achieved due to the self-weight of the replacement material (Nishida and Nakazawa, 1991). Coarse-grained sands or crushed stones, which do not need compaction, are used as replacement materials. However, this procedure has some limitations such as non-uniform replaced section and incomplete removal of soil layer.

(b) Displacement

Typical case of Displacement refers to the improvement by Sand Compaction Pile (SCP) Method (For details, refer Section 4.3), in which the replacement rates of sand pile are densely arranged. The soft ground will be pushed laterally and displaced by driving sand piles. In this case, 60～80 % of replacement rate for SCP are desirable.

(c) Explosive Displacement

If some explosive is inserted in the objective soft soil under surcharging mound and it is burst, the replacement of soft soils may be accelerated by the gas generated during the blasting process. At the same time, soft soils are replaced with suitable soils, which are filled as replacement. The replaced soils are compacted by the gas pressure and the vibrating power by the blasting. Depending on the locations of the explosives, the explosive displacement is named differently: Trench-shooting style, Toe-shooting style and Under-filling style. Fig. 3.5 shows the procedures of Under-fill style as a representative example of the above-mentioned 3 types (Nishida and Nakazawa, 1991).

3.2.4 Planning

The procedures of Replacement Method generally involves the following 3 steps.

Step 1: Determining the depth, the width and the length of the removed soil stratum.

Step 2: Determining the engineering properties of replaced soils.

Step 3: Numerical analysis for determining the allowable settlement and bearing capacity of superstructure to be constructed, and stability analysis against slip failure.

The typical flow of design for Replacement method is shown in Fig. 3.6 (Nishida and Nakazawa, 1991).

3.2.4 Applications

Fig. 3.7 shows an application of Replacement method, which was adopted for the ground improvement in the Tokyo International Airport Expansion Project from 2007 to 2008 (Horiuchi et al., 2008). In this case, Replacement by underwater excavation and Displacement by Sand Compaction Pile were adopted. The soft clay layer was distributed within an average thickness of 35m from the sea bottom, for which the settlement was estimated to be 8.0m. Revetment bank with a gentle sloping style was adopted in the design to prevent any ground settlement, and also to conserve the environment for floras and faunas.

In order to prevent slip failure of the covering embankment of the sea side slope, Replacement method was adopted at the toe of slope, where soft soils were excavated and removed, and replaced those with sandy soils. In Sand compaction pile (SCP) method, 30% replacement rate was applied for improving the soft clayey soils under the revetment bank. Displacement by sand compaction pile with 60% replacement rate was applied on either side of the slope toe.

3.3 Preloading using fill and vacuum

3.3.1 Description

When a structural load is placed on the highly compressible and soft ground, settlement of ground surface may occur. If the bearing capacity of the ground is not sufficient for that load, ground failure will take place. Preloading Method is one of ground improvement techniques to prevent these kinds of troubles in the foundation soils.

This is a method in which the soils is consolidated by constructing temporary embankments, which apply a surface pressure equal to that of the structural pressure after construction is completed. The embankment is removed once the desired settlement is reached. As the settlement progresses, the void ratio of the soil decreases, resulting in the increase of density, which increases the soil strength.

The steps involved in preconsolidation are as follows.

- Preload the area and induce consolidation prior to construction.

- Sometimes, vacuum is applied in soft clay to induce negative pore water pressure, and thus increasing the effective stress. The increased effective stress works as a preloading.

3.3.2 Principle and Mechanism

Detailed description on the phenomenon of consolidation has already been given in “Chapter 1.1.2 Soil mechanics aided ground improvement”. The decrease of the void ratio due to loading is shown in Fig. 1.5 in the form of e-log p curve. The entire processes of the loading and unloading of dirt and the building construction are plotted in an e-log σ diagram as seen in Fig. 3.8. Point A is the starting point at the site before construction. During the preloading process of the dirt with Δσpreload, it moves from Point A to B. During removal of the dirt, it moves back to Point C. During the building construction process with Δσbuilding, it moves back again from Point C to Point D. When the resulting changes in void ratio are compared in the cases of with-preloading (Δewith preload) and without-preloading (Δew/o preload), it is clear from the figure that Δewith preload is much smaller than Δew/e preload. Thus, consolidation settlement during building construction will be considerably reduced.

The underlying principle of this type of ground improvement can be described as following.

- Loading on the ground surface generates excess pore water pressure in the ground.

- Groundwater trends to flow to low pressure zone

- Soil water escapes

- Consolidation and settlement will progress

- Soil density will be increased

- Soil strength will be increased

Fig. 1.6 shows the process under the condition of both top and bottom drainage. Fig. 1.10 and Fig. 1.11 show the mechanism of the soil strength increase due to consolidation. Equation (1.15) indicates the Time factor Tv for the estimation of consolidation.

To estimate the soil strength increase due to consolidation, Skempton (1957) proposed the following Equation for the increasing rate of soil strength.

cu／p＝0.11+0.0037Ip　　　         (3.3.1)

where:  cu: cohesion

p: consolidation pressure

Ip: plasticity index

For the preliminary estimation, the value c/p adopted ranges from 1/3 to 1/4.

Vacuum consolidation technique

The method to act vacuum on the ground instead of the consolidation pressure was first proposed by Kjellman in 1948, and was later popularized by Japanese Company (Penta-Ocean Co. 1968). However, it is difficult to maintain the vacuum condition continuously, and as a result, there were not many actual applications of this technique. Recently, a new vacuum consolidation technique has been developed. This technique induces a vacuum in soft clay through pipes in a sealed environment using impermeable membranes at the top (refer to Fig.3.11 in later Section). Typically, 80 kPa (-78% of the atmospheric pressure) or greater vacuum (negative pore water pressure) can be applied by this method. Because of the induced vacuum pressure in the soil mass, the atmospheric pressure works the same way as in the preloading technique through the impermeable membrane on top of the ground. An 80 kPa vacuum pressure is equivalent to about 4.0 m height of soil load in the preloading technique.

3.3.3 Procedure

When applying load through embankment construction (Fig.3.9), the procedure is the same as in the usual filling of soils (refer Section 2.2). If the bearing capacity of treating ground is not sufficient enough to support the designated embankment, the step-by-step filling method may be adopted.

In the case of Vacuum consolidation method, vacuum pipe lines are installed, on the top of which a sand mat is spread and is covered with airtight membranes. The operation of the vacuum pump will be through the vacuum pipe lines, and ultimately vacuum will be acting in the sand mat.

To promote the consolidation effects, Preloading method is usually combined with Vertical drain method, which is described later in this chapter.

Designing and applications on “Preloading using fill and vacuum” are omitted, because they are similar to those of ”Vertical Drain Method”, described in section 3.4 below.

3.4 Vertical Drain Method

3.4.1 Description

Preloading Method (as described in 3.3) is one of the ground improvement techniques, which helps to drain out pore water in the ground artificially, and, thereby, subjecting the ground to consolidate, and thus, improving the compressibility and shear strength of ground. However, when this method is applied to ground with thick layers, the time to achieve the final consolidation settlement will be longer. To overcome this problem and accelerate the consolidation, Vertical Drain Method was developed.

A large number of vertical drains are arranged in the targeted soft ground. The underlying principle of this method is that the consolidation time can be substantially reduced by changing the direction of drainage into horizontal, by which the drainage distance can be shortened through installation of many vertical drains. A typical example of Vertical Drain Method, Sand Drain Method is shown in Fig. 3.10. In this case, it is necessary that the sand mat be laid on the ground surface and the surcharge (filling or embankment) is applied onto the sand mat. This surcharge will generate excess pore water pressure in the ground, which promotes the drainage.

The classification of this method is shown in Table 3.2 according to the materials used for the drains.

Table 3.2 Types of Vertical Drain Method

| Name | Materials of Drain | Size of drain |
| --- | --- | --- |
| Sand Drain Method Packed Drain Method Prefabricated Drain Method | Sand pile in-situ placement Sand pile covered by synthetic fiber Card board, Plastic board, Fabric board, Compounds of wick and unwoven filter | φ:40 – 80cm φ:10 – 20cm (Standard 12cm) rectangular (5 - 10cm width and 0.3 – 1.0cm thickness) |

When filling is not readily available for site conditions or shear strength of the soft soil is insufficient for supporting of the surcharge, an alternative means of inducing consolidation is to apply a vacuum to the surface through a system of the vertical drains called the Vacuum Consolidation Method (Fig.3.11), which is already discussed in the previous section.

3.4.2 Principle and Mechanism

Using the one dimensional theory of consolidation, the time factor equation (1.15) can be rewritten as

(3.4.1)

Equation (3.4.1) implies that consolidation time t is proportional to H2, where H is the longest drainage distance. The techniques call to shorten H in the field with inserted vertical drainage materials. The drainage materials may be sand columns, plastic prefabricated wicks, or paper and card boards as mentioned above. As seen in Fig. 3.12, stripes of vertical drains are arranged such that the drainage distance Hd is much shorter than the original drainage distance H/2 without vertical drain installation. For example, if Hd to H/2 ratio becomes 1/5, the consolidation time reduces by 1/25, according to Equation 3.4.1. In recent years, wick drain has started replacing paper drain in most applications. The wick is made of synthetic materials and it consists of a long core material in the center with drain ribs in the longitudinal direction and geo-fabric filter material around it, as seen in Fig. 3.13.

3.4.3 Procedure

In the case of Sand drain method, drains are installed as follows (Fig. 3.14):

- A casing pipe of the required diameter with its bottom closed in a loose-fit-cone is driven up to the required depth.

The cone is slightly separated from the casing by driving a mandrel into the casing

- The sand of the required gradation is poured into the pipe for a short depth and at the same time the pipe is pulled up in steps. As the pipe is pulled up, the sand is forced out of the pipe by applying pressure on to the surface of the sand. The procedure is repeated until the hole is completely filled with sand.

The drains may also be installed by jetting a hole in the soil or by driving a casing into the soil, washing the soil out of the casing, and filling the hole with sand afterwards.

It is possible that sand drains may not function satisfactorily if the soil surrounding the well gets remolded. This condition is referred to as “smear”. Although theories have been developed by considering different thickness of smear and permeability, it is doubtful whether such theories are of any practical use since it would be very difficult to evaluate the quality of the smear in the field.

In the case of Prefabricated vertical drain method, the shape and materials of the vertical drain are prefabricated and are then marketed. Some typical cross-sectional forms of wicks are shown in Fig. 3.15.

The following are the advantages of prefabricated vertical drain as compared with the sand pile (Ingles & Metcalf, 1972).

- the rapidity with which a drain can be inserted – a typical production time for 20m depth is 3 mins,

- the strength of the strip ensures continuity of drainage irrespective of the settlement although the introduction of sand drains prepacked into porous plastic tubes also overcomes discontinuity problems,

- the drains can be closely spaced to allow rapid consolidation.

In general, drains are placed to the depth of 20 to 25 m with spacing of 1 m to 2 m.

The typical illustration of the PVD driving machine is shown in Fig. 3.16.

The end of roll of strip is attached to a mandrel guided by a steel rod. Due to the self-weight of the rod and assisted by vibrator to the top of the rod, the mandrel penetrates to the required depth at a rapid rate. The mandrel is then withdrawn. The whole unit is mounted on a tracked vehicle equipped with a mast of sufficient height to guide the mandrel rod.

3.4.4 Design

The fundamental equation of consolidation on Sand Drain Method is given below.

（3.4.2）

where,  u : excess pore water pressure

t：consolidation time

cvh：coefficient of consolidation in horizontal direction

r：distance from the center of sand pile to the boundary

The solution of this equation is obtained from the relationship between the average consolidation degree U and the time factor Th, and it is shown in Fig. 3.17 (Takagi, 1955).

When the vertical drain is adopted, the consolidation time is dominated by the drainage in the horizontal direction, and therefore, the required time for consolidation is calculated by the following equation.

(3.4.3)

where, t：required time to reach degree of consolidation U（day）

Th：time factor of horizontal direction

(its value for a given consolidation degree is read from Fig. 3.18)

de：effective drainage diameter of sand pile (cm)

cvh：coefficient of consolidation on horizontal direction (cm２/day）

The effective drainage diameter de of sand pile is obtained from the inflow sphere of the pore water to each sand pile (diameter dw). Expression for de differs depending on the arrangement of sand piles (Fig. 3.18).

In case of squared arrangement of sand piles (Fig. 3.19 (a))

(3.4.4)

where: d: distance from center to center of each sand pile

(3.4.5)

In case of equilateral triangle arrangement of sand piles (Fig. 3.19(b))

Symbol n for each curve in Fig. 3.18 is the ratio of the effective drainage diameter (de) and the diameter (dw) of each sand pile (Fig. 3.18). The subscript of symbols in the equation “h” means horizontal direction. Since the coefficient of consolidation on horizontal direction, cvh cannot be obtained from the conventional consolidation test, here the coefficient of consolidation cv in the vertical direction is used. However, it is recognized that the relation of cv and the cvh may be cvh = (1 - 5)cv, the above-mentioned designing will be the reviewing of the safe side.

Detailed descriptions of Sand drain are described in Murthy book (2002).

In case of wick drains, it is first necessary to estimate an equivalent sand drain diameter for the wick drain used. The equations suggested by Koerner (1999) are

(3.4.6)

(3.4.7)

where  dsd: equivalent sand drain diameter = dw

dv: equivalent void circle diameter = de

b,t: width and thickness of the wick drain

ns: porosity of sand drain

nd=(Void area of wick drain/total cross sectional area of strip)

=(Void area of wick drain/b×t)

It may be noted here that equivalent sand drain diameters for various commercially available wick drains vary from 30 to 50 mm.

The equation for estimating the time t for consolidation is

(3.4.8)

where  ch: coefficient of consolidation of soil for horizontal flow

d: equivalent diameter of strip drain

=(circumference/π)

D: sphere of influence of the strip drain (see Fig. 3.19)

U: average degree of consolidation

General design procedures of Sand Drain Method are shown below:

- The diameter of sand pile, its arrangement and their mutual distance are determined with reference to the particular conditions in the site (de and n are obtained).

- Coefficient of consolidation is determined from the results of laboratory tests and soil investigations.

- The target degree of consolidation (U) is determined (usually U=80-90%).

- Time factor Tv is obtained using Fig. 3.17).

- Substituting each numerical value in Equation 3.3.3, the time for consolidation to reach the assumed degree of consolidation can be calculated.

3.4.5 Application

Vertical Drain Methods were put to practical use during the 1930s to the 1940s. An application on Sand Drain was reported at 1933, and it of Prefabricated Vertical Drain was reported at 1948 as Cardboard Drain Method (JGS, 2006). Thereafter, many applications (roads, railroads and marine) of them were reported all over the world.

In recent years, an example that this method was applied to the huge artificial and offshore island at the Kansai International Airport project in Japan (Furudoi and Kobayashi, 2009). The outline of the project is shown in Table 3.3 and Fig. 3.19.

Table3.3 Outline of Kansai International Airport Project

| Items | Contents |
| --- | --- |
| History | 1987 Starting of first phase project 1994 Opening of Kansai International Airport (One runway ) 1996 Starting of second phase project 2007 Start of two runway serviceability |
| Location & Scale | 5km offing from the coast, the seabed at a depth of 18 – 20m First phase: 510ha (average seawater depth: 18m) Second phase: 545ha (average seawater depth: 20m) |
| Geological condition | Soft Holocene clay layer with 20 – 25m thickness Hundreds of maters of the Pleistocene clay layer under-laid |
| Sand pile’s number to be adopted | First phase: one million (diameter: 400mm) Second phase: 1.2 million (diameter: 400mm) |

Alluvial clay layers (Holocene) with the thickness of 20-25 m were deposited under the seabed ground to be constructed. It was anticipated that large ground settlements might occur during construction of the airport and after its completion. Sand Drain Method was adopted as a countermeasure, and sand piles (diameter: 40 cm) were arranged in a rectangular pattern (1.5 m×2.5 m )under the structures such as seawalls, and square pattern (2.5 m×2.5 m) under landfilling (Fig. 3.19).

References

Barron. R.A. (1948): Consolidation of Fine-grained Soils by Drain Wells, Transactions of ASCE. Vol.113, pp718-724 Paper No.2346.

Chu, J., Varaksin, S., Koltz, U. and Menge, P. (2009): Construction Processes, Proceeding of the 17th International Conference on Soil Mechanics and Foundation Engineering, pp.3006-3135.

Furudoi, T. and Kobayashi, M. (2009): Geotechnical Issues and Approach on Kansai International Airport Project, - Prediction and Performance of Settlement-, Journal of Japan Society of Civil Engineers, C, Vol. 65, No. 4, 998-1017.

Geo-engineering Co. (http;//www.sp-kouhou.com/sp/#PAGETOP), Brochure.

Horiuchi, D., Aikawa, S., Mukubayashi, K. and Ito, Y. (2008): Ground Improvement for D-Runway and Reclaimed land, Construction Project of D-runway of Tokyo International Airport, The 5th Technical Meeting, Tokyo Airport Improvement Office of MILT, Conference Materials, (In Japanese).

Ingles. O.J ＆ Metcalf. J.B. (1972): Soil Stabilization, Principle and Practice, Butterworth, UK.

Japan Geotechnical Society (2006): Terminology and Dictionary on Geo-technology, (In Japanese).

Kjellman, W. (1948): Accelerating consolidation of fine-grained soils by means of cardboard wicks, Proc. of the 2nd ICSMFE, Vol. 2. 302-305.

Murthy, V.N.S. (2002): Principle of Soil Mechanics and Foundation Engineering, CRC Press, pp.974-979, US.

Nishida, K., and Nakazawa, J. (1991): Replacement Method, Manual of Ground Improvement Method, Nikkan Kogyo Shimbun, pp.127-138, (In Japanese)

Penta-Ocean Construction Co. Ltd. (1968): Paper Drain Method and Vacuum Consolidation Method, Rikoh Tosho Publ. Japan (In Japanese).

Takagi, S. (1955): Graph for sand drain drainage and its application, Soils and Foundations (Journal of JGS), Vol.3, No.12, p.8, (In Japanese).

Skempton, A.W.(1957): Discussion on the Planning and Design of the New Hong Kong Air Port, Proc. of Institution of Engineers, Vol. 7, pp.305 – 307.

Questionnaires – Practicing what you have learned.

3.1 Does vibro-replacement works good on silty and clayey fine sand and is it cheaper option of ground improvement compared to piles?

3.2 About vertical drain such as sand drain, pvd drain. What are the settlement, efficiency, consolidation characteristics of fiber reinforced sand drain