Answers and Suggestions to Questionnaires.

Chapter 1

1.1 How is soil a very unique material and complex in nature?

Answer: The unique characteristics of soils are as follows:

- It is not solid continuous material, but is rather composed of three different constituents – solid (grain), water, and air – and is thus aggregated material.

- Particle sizes have significant influence on soil behavior from granular soil to clay.

- The amount of water also plays a very important role in soil behavior.

- Its stress – strain relation is not linear from the small strain levels.

- Its pore space possess the capability of water flow.

- It has time – dependent characteristics; that is, it is creep susceptible.

- It swells when wetted or shrinks when dried.

- It is an anisotropic material due to the particle shapes and the depositional direction under gravity.

- It is also spatially nonhomogeneous.

1.2 Summarize for the following questions on Ground improvement techniques.

1.2.1 What is the engineering purpose of adopting Ground improvement techniques?

1.2.2 What principles are Ground improvement techniques?

Suggestion: Refer to Section 1.1.

1.3 The engineering strength τ of soil is expressed by the cohesion c and the internal friction angle φ, but consider the effect by increments of c and φ according to the principle of ground improvement.

Suggestion: Refer to Section 1.3 and Table1.1.

1.4 What are the difficulties faced with soft clay?

Answer: Soft clays represent a well-known category of problematic soils which are generally encountered under the form of deposited layers in coastal areas. Several problems are faced when dealing with the study of soft clays from field investigation, soil characterization, behavior modeling, and stability of geotechnical structures up to ground improvement solutions.

1.5 How is loose sand a challenging soil?

Answer: Quicksand forms in saturated loose sand when the sand is suddenly agitated. When water in the sand cannot escape, it creates a liquefied soil that loses strength and cannot support weight. Quicksand can form in standing water or in upwards flowing water (as from an artesian spring). In the case of upwards flowing water, seepage forces oppose the force of gravity and suspend the soil particles.

1.6 What are the need for improving the ground?

Answer: 1. Increase of strength, 2. Reduce distortion under stress (Increases of stress-strain modulus), 3. Reduce compressibility (volume decreases due to a reduction in air voids or water content under loads).

Chapter 2

2.1 What is compaction? When is it adopted?

Answer: In geotechnical engineering, soil compaction is the process in which a stress applied to a soil causes densification as air is displaced from the pores between the soil grains. When stress is applied that causes densification due to water (or other liquid) being displaced from between the soil grains then consolidation, has occurred. Normally, compaction is the result of heavy machinery compressing the soil, but it can also occur due to the passage of (e.g.) animal feet.

2.2 What changing factors of engineering properties are there in the compacted soil by Compaction method?

Answer: If soil materials with suitable water contents are compacted, the void ratio of soil mass is decreased and the dry density is increased. As a result, the engineering properties of soil mass will be improved.

That is;

- The shearing resistances (c, φ) and California Bearing Ratio, etc. will be increased.

- It prevents engineering troubles such as the compression settlement and the submerged settlement of soil structures.

- When the seepage control effect for soil structures is intended such as the core of earth fill dams, the permeability can be lowered.

2.3 Why do we should draw the zero air voids with the compaction curve?

Answer: ZAVL corresponds to degree of saturation = 1, and no matter how much energy you give there is no state possible beyond ZAVL. In other words, it is not possible to compact the soil beyond the ZAVL.

2.4 What is Vibro-compaction? In which soils is it adopted?

Answer: Vibro-compaction is a ground improvement technique that densifies clean, cohesionless granular soils by means of a downhole vibrator. It is used to control and reduce settlement, mitigate liquefaction, stabilize or treat hydraulic fill and limit lateral pressure behind quay walls.

The vibrator is typically suspended from a crane and lowered vertically into the soil under its own weight. Penetration is usually aided by water jets integrated into the vibrator assembly. After reaching the bottom of the treatment zone, the soils are densified in lifts as the probe is extracted.

要　英文チェック

2.5 Vibrating power is effective on the densification of loose sandy ground. Moreover, it is considered that pouring water is more effective during Vibrating. Answer how principle it becomes more effective and what name of the ground improvement method is utilizing this principle.

Suggestion: Refer to Section 2.3

2.6 What is vibro-flotation? What are the applications of vibro-flotation?

Answer: The Vibroflotation (vibrocompaction) is a method of improvement of non-cohesive soils by re-arranging the grain distribution pattern while applying cyclic vibrations which cause the outflow of granular soil. As a result, the compaction of soil and pore volume reduction are obtained.

This is technique used for stabilizing granular soils such as loose sand, gravel and backfill material. The method consists in using vibration to create a suspension with the particles so that they can be rearranged to form more dense materials.

2.7 What are the merits and demerits of dynamic compaction?

Answer: As following

Advantages:

*Versatile technique, economical and fast,

*Very sustainable technique: no additional materials, no cement, no water is required,

*Well adapted to small & large scale projects,

*Real-time adjustment of the applied energy to the actual ground conditions,

*Flexible with changes in the locations and dimensions of footing,

*Improve bearing capacity of soil, reduce settlement and differential settlement in addition to mitigation of soil liquefaction,

*Improve the entire site including building, roads and infrastructure.

Disadvantages:

Dynamic compaction involves lifting and dropping a heavy weight several times in a one place. The process is repeated on a grid pattern across the site.

By Menard

2.8 What is Heavy tamping (Dynamic compaction)?

Answer: The heavy tamping achieves deep ground densification using the dynamic efforts of high energy impacts resulting from the drop of large pounders. The basic principle of the technique is a transmission of high energy waves through a compressible soil layer in order to improve its geotechnical properties at depth. This is applicable to any type of granular soil. This technique is particularly well-adapted to nonorganic heterogeneous fill, made ground and reclamation areas with varying characteristics, even with the presence of lar4ge blocks. Heavy tamping is effective in both unsaturated and saturated soils below the water table.

Chapter 3

3.1 Does vibro-replacement work well on silty and clayey fine sand and is it cheaper option of ground improvement compared to piles?

Answer: Some "conditions" must be defined before answering your question.

1. What is the range and character of loads that must be transferred to the ground?

2. What are the requirements concerning settlements of supported structure?

3. Are there any environmental limitations for applied foundation technology?

Vibro Replacement Columns (VRC) have a double function:

- create a vertical drain within cohesive soil and fasten the consolidation,

- form a rigid inclusion (stone column) that increases the stiffness of the soil body.

So, answering your question, I'd give some recommendations:

- You may use VRC if no restrictions are established for the vibrations.

- Due to limited internal capacity, VRC cannot substitute concrete piles when large loads are applied. The application of VRC in very soft soils (and in peat) is also limited for the same reason.

- Limited capacities may result with large size of capping slabs (decrease of load for a singular column).

- Consolidation takes some time and imposes settlements. Relatively long time of construction or slow increase of loads is recommended. Otherwise, supported structure should be somehow "adjustable".

- The good thing is that in the case of large infrastructural projects, when the loads are distributed over large areas and increase slowly, VRC are very beneficial. Not only because of lower cost compare to concrete piles, but also because of long-term improvement of surrounding soil layers.

3.2 When is preloading adopted as a ground improvement techniqu

Answer: The most common methods of ground treatment in cohesive soil to reduce the void ratio or water content of soil by pre-consolidation. This increase the soil and reduce the compressibility is commenced.

3.3 Describe basic principle of Vacuum Consolidation Method

Answer: Cohesive soils (clayey soils) are generally characterized by having a low permeability. Consolidation of a cohesive soil relies on the ability to evacuate water out of the soil matrix and this process can be long. Vacuum Consolidation Method is an atmospheric consolidation system used to accelerate consolidation of cohesive, highly compressible soils. The process consists in creating a negative pressure under an airtight membrane laid over the ground, which generates atmospheric pressure on the ground, equivalent to the pressure by a 4 meter embankment. This method can substantially reduce consolidation time thanks to early pre-loading as soon as the system is in place and without risk of ground failure. The procedure consists of installing a vertical and horizontal draining system combined with a vacuum pumping system under an airtight impervious membrane.

3.4 What are vertical drains and why do we use them?

Answer: When a load is applied on saturated cohesive compressible soils (clays, silts), the structure can settle for long period of time (up to 10 to 20 years). Vertical drains are a technique which is used to control long term residual settlement in these type of soils within acceptable limit.

3.5 What is the function of vertical drain?

Answer: Low permeability, the consolidation settlement of soft clays takes a long time to complete. To shorten the consolidation time, vertical drains are installed together with preloading either by an embankment or by means of vacuum pressure. Vertical drains are artificially-created drainage paths which into the soft clay subsoil. Thus, the pore water squeezed out during consolidation of the clay due to the hydraulic gradients created by the preloading, can flow faster in the horizontal direction towards the vertical drains. It is taken advantage of the fact, that most clay deposits exhibit a higher horizontal permeability compared to the vertical.

3.6 What is advantage of using vertical drains along with pre-loading?

Answer: The purpose of preloading and vertical drains is increase the shear strength of the soil, to reduce the soil compressibility and to reduce the permeability of the soil prior to construction and placement of the final construction load and prevent large and/or differential settlements and potential damage to the structures.

3.7 Compare sand drains and wick drains.

Answer: A sand drain is basically a hole drilled in a cohesive soil and filled with sand. Since the sand has larger particle size, its permeability is much higher, thus water will flow through it much more easily. As shown above, an array (it’s actually a two-dimensional array) of sand drains is installed, and load is applied on top of the drains. The load shown above is an embankment, such as is used on a highway, and an additional, or surcharge, load is used to speed up the drainage process. The excess water is collected at the top and directed away from the jobsite.

A wick drain is just what the name implies: a geo-synthetic “rope”, usually about 100 mm wide and 5 mm thick, which acts as a high-permeability conduit for water to flow out of the soil and to the surface, in the same manner as takes place with sand drains. As is the case with sand drains, they are installed as an array, generally in 3 meter spacing.

3.8 About vertical drain such as sand drain, pvd drain. What are the settlement, efficiency, consolidation characteristics of fiber reinforced sand drain

Suggestion: pl find attachment.

http://file.scirp.org/pdf/OJSS20120300013_15601846.pdf

Article Strain hardening behaviour of fibre-reinforced sand in view o...

Chapter 4

4.1 How is stabilization of soil achieved by cement?

Answer: Cement stabilization of soil is done by mixing pulverized soil and Portland cement (cementitious solidifying material) with water and compacting the mix to attain a strong material. The material obtained by mixing soil and cement is known as soil-cement. The soil cement becomes a hard and durable structural material as the cement hydrates and develops strength.

4.2 What are the various stages of action in lime stabilization?

Answer:

*Scarification and Initial Pulverization

*Lime Spreading

*Preliminary Mixing and watering

*Final mixing and pulverization

*Compaction

*Final curing

4.3 Is the technique of geosynthetic-encased stone columns in soft clay applicable really? And is it possible to make geosynthetic around a stone column technically?

Answer: Please have a look at the following articles:

1.Almeida, M.S.S., Hosseinpour, I., Riccio, M., and Alexiew, D. (2015). “Behavior of Geotextile-Encased Granular Columns Supporting Test Embankment on Soft Deposit.” Journal of Geotechnical and Geoenvironmental Engineering, 141(3), 04014116.

2.Mohapatra, S.R., Rajagopal, K., and Sharma, J. S. (2016). “Large direct shear load test on geosynthetic encased granular columns.” Geotextiles and Geomembranes 44(3), 396-405.

4.4 Is flyash a good choice for stabilizing dispersive soil?

Answer: Since this is a railway embankment, please read carefully UIC 719R recommendations.  Rail embankments differ from road embankments primarily in higher stiffness and bearing capacity characteristics.  The materials used for the body of the embankment depend on climatic and transport conditions.  The preferred materials are normally of no or low plasticity and with fines not greater that 35% for easier laying and achieve high compaction. The higher the design speed and axial load, the better (and coarser) materials have to be used.  Fly ash enhanced expansive clay soils are at the bottom end of the scale so please make sure that the resultant material when compacted complies with the min CBR and Plate loading tests (Ev2 >= 45MN/m2 for fine soils, Ev2/Ev1 <= 2.2).  Sandwiching such fine grained materials between coarser layers is recommended.

4.5 What are the influences of chlorine on a cement stabilized sand / soil?

Answer: Salts such as sodium chloride and calcium chloride can increase the strength of cement-stabilized soils.

要　英文チェック

4.6 Describe the differences between Sand Drain Method and Sand Compaction Pile Method.

Suggestion: Refer to Section 3.4 and 4.3

By Menard

4.7 What are stone columns and why do we use them?

Answer: Like most ground improvement techniques, stone columns are used to reduce settlement and increase load-bearing capacity. They also accelerate soil consolidation as a result of drainage capacity of the granular material within the columns, which act as pore or water pressure evacuation points. Stone columns are particularly effective in improving slope stability and preventing liquefaction by increasing shear strength within a soil. These vertical inclusions can be made of stone or sand and are installed in a grid pattern into the soft soils beneath the structure. Stone columns also known as aggregate piers are well suited for the improvement of soft or loose soils as they create vertical inclusions with high stiffness, shear strength and draining characteristics. The Stone Columns result in a increased bearing capacity, and a reduction of the total and differential settlements.

Chapter 5

5.1 Define grouting materials

Answer: Grouting materials are generally mixtures of water, cement, silicate or other chemical materials. They are applied as a thick emulsion and hardens over time, much like its close relative mortar.

5.2 Write the applications of grouting.

Answer: As following,

*Stopping major water inflows in mines, tunnels, parkades, dams and underground structures in general

*Soil and rock grouting

*Environmental applications

*Rehabilitation of deteriorated, porous or leaking concrete or masonry structures, with advanced techniques, using the most suitable technology and grouts.

*Shoring and retaining systeems

*Special foundation and retaining systems

*Trouble Shooting

*Soil stabilization

*Mining Applications

*Unusual and difficult geotechnical and structural problems

5.3 What are the various methods of grouting?

Answer: The process of grouting consists of filling pores ort cavities in soil or rock with a liquid from material to decrease the permeability and improve the shear strength by increasing the cohesion when it is set. Cement base grout mixes are commonly used for gravely layers or fissure rock treatment. Chemical solutions grout are used for sandy ground.

5.4 浸透注入工法の施工方法と使用する硬化剤のゲルタイムの関係を示せ。

Show the relation between Grouting procedures and Gel Time of chemical solutions on Permeation Grouting.

Suggestion: Refer to Item 5.2.3.

5.5 In ground improvement, workability (or flow-ability) for wet soil mixing technique is related to mixes limit of liquidity.

So, I would like to know how this should be done.

Answer: One of the measures to determine the workability is slump testing which is often used prior to the concreting of diaphragm wall or superstructure. By the slump testing, you will be able to see whether the concrete adopted can flow far enough.

5.6 Jet GroutingとDeep mixing methodの違いを説明せよ。

Describe the differences between Jet Grouting and Deep Mixing Method.

By Menard

Suggestion: Refer to Section 5.3 and 5.4.

5.7 What is jet grouting and why do we use it? Describe basic principle of it.

Answer: Jet grouting is a soil reinforcement technique. It is as a targeted treatment for wide range of applications: control and reduce settlement under structures, increase of bearing capacity, create impervious cut-off wall or impervious bottom for deep excavation, install retaining walls, underpin existing structures, reinforce soils with existing utilities lines and burred structures, operate in areas that difficult to access (limited headroom, cramped spaces, etc.). Jet grouting improves the mechanical characteristics of the soil using a fluid jetting with very high kinetic energy that breaks up the soil structure and mixes the soil particles in-situ with a grout to create a homogeneous mass of high strength reinforced soil-cement materials.

By Menard

By Menard

5.8 When should jet grouting not be used?

Answer: As a rule, jet grouting suits poorly to soils with blocks and rough gravel, to organic materials. It is not adapted either to soils through which large volumes of water flow.

By Menard

5.9 Is the binder injected always in liquid form?  (Soil Mixing)

Answer: The binder is not always injected in liquid form. It is also possible to inject the binder either as liquid slurry or as a dry powder. The liquid slurry is injected by means of a pump (wet technique). The dry powder is injected into the ground by means of compressed air (dry technique). The water in the ground generates hydration and concrete setting.

By Menard

5.10 What is Soil Mixing (Deep Mixing) and why do we use it?

Answer: Soil Mixing is a ground improvement technique. It is used for a wide variety of applications: controlling and reducing settlement under structures, increasing bearing capacity of the soil, ensuring stability, reducing liquefaction risk, mass stabilization, reducing earth pressure behind retaining structures, block ground water, increasing lateral reaction around foundation piles, etc. Soil mixing consists of specially designed augers or special tools to mechanically mix with an in-situ binder. The process simultaneously breaks up the soil without removing it, injects a binder at low pressure and thoroughly mixes the binder with soil to form a reinforced block of soil after treatment.

Chapter 6

6.1 Write the concept behind soil reinforcement and define geosynthetics.

Answer: To resist the erosion of the soil during the rainy season and also to give strength to loose soil.

Geo-synthetics are human-made materials made from various types of polymers used to enhance, augment and make possible cost effective environmental, transportation and geotechnical engineering construction projects. They are used to provide one or more of the following functions: separation,. Reinforcement, filtration, drainage or liquid barriet..

6.2 What is soil reinforcement?

Answer: Reinforcement placed in horizontal layers throughout the height of the wall provides the tensile strength to hold the soil together. The reinforcement materials of  MSE can vary. Originally, long steel strips 50 to 120 mm wide were used as reinforcement. These strips are sometimes ribbed, although not always, to provide added friction. Sometimes steel grids or meshes also used as reinforcement. Several types of geo-synthetics can be used including geogrid and geotextiles. The reinforcing geo-synthetic can be made of high density polyethylene, polyester, and polypropylene. These materials may be ribbed and are available in various sizes and strengths.

6.3 Write a brief note on geo-synthetics as reinforcement.

Answer: Geogrids or geotextiles can be used as reinforcement to increase shear strength of soils, thereby providing a more component structural material. Example of this application include the use of geogrid to reinforce a steep slope, or to strengthen a base course in a pavement system.

6.4 What is the most effective way to Reinforced fine soils like clay?

for example could strip reinforcement used for fine soils? As reinforced walls should be backfilled with coarse materials like coarse sand or gravels

Answer: I suggest to find Brazilian publications. Is common to us use fine soils to build reinforced walls.

Dr. Karaman has suggested an interesting instrumented GRS. In the sequence are others that you can read.

Regarding your questions, I do not recommend to use strips in fine soils because of the interface shear ｓｔｒｅｎｇｈ. I also do no recommend to use vertical face walls (84° or 10V:1H is recommended).

6.5 Can we remove OH compound from any fiber material chemically?

Answer: I want to use some fiber as a soil reinforcement, for the purpose of soil stabilization. There is only one limitation that it is water absorbent, which may lead to shrinkage. That is harmful for the structure. Although it has other excellent properties which is in favor of soil stabilization. Tell me whether it is possible. With the detailed procedure

6.6 Define soil nailing.

Answer: Soil nailing is an earth retention technique using grouted tension-resisting steel elements (nails) that can be design for permanent or temporary support. The walls are generally constructed from the top down. Typically, 1 to 2 m of soil is excavated from the top of the planned excavation. Near-horizontal holes are drilled into the exposed face at typically 1 to 2 m centers. Tension-resisting steel bars are inserted into the holes and grouted.

Chapter 7

7.1 Why are the clay surfaces charged negatively? What is the importance of the absorbed water layer around clay surface?

Suggestion: Refer to Item 7.2.2

7.2 What is electric-osmotic dewatering?

Answer: Dewatering techniques of dewatering done through the use of cathodes and anodes with passage of Electrical current. Electro-osmosis is defined as “the movement of water (and whatever is contained in the water) through a porous media by applying a direct current (DC) field”. It is the only effective method of dewatering in deep clay soils.

7.3 What differences are there in the freezing system on Ground Freezing Method?

要　英文チェック

Suggestion: Refer to Item 7.3.3.

7.4 Consider what contribution Ground Improvement Techniques makes possible to SDGs.

Suggestion: Refer to Item 7.4.1 and 7.5.1

7.5 What is bio technical ground improvement?

Suggestion: Refer to Section 7.4.

7.5 Is there any industrial waste material that can improve engineering properties and reduce swell-shrink behavior of expansive soil by stabilization?

Answer: 1) You can use GGBS, Cement kiln dust, fly ash, granite dust, mill scale, polyvinyl waste, silica fumes for stabilizing expansive soils

2) fly ash from industrial waste would be the first choice. But you can try Biomass ash such as sugar cane, rice husk. They are environmental friendly choice.

3) Hydrated lime is cheap and effective .However it is not considered as industrial waste material.

7.6 What are uses of waste in geotechnical engineering?

Answer: Waste use in improving soil engineering specification such as disaster waste, cement waste and others

There are several studies that analyze the use of residues for soil stabilization, among them: the ashes (sugar cane bagasse, rice basket), fibers (sugar cane bagasse, coconut, polymeric), construction waste, stone dust, and glass dust, among others.

The uses of waste depends on the nature and property of the waste.

Improvement of soil strength

Reduction of soil swelling

Reduction of soil settlement

Improvement of soil elasticity

Soil Erosion resistance improvement

Soil Liquefaction resistance improvement

Reduction/ Improvement of soil permeability

7.7 Describe definitions and applications of Disaster waste.

Suggestions: See Section7.5.3 and Section 7.5.5. 2)